BAO CAO TIM HIEU

Su khac nhau giua X va x trong GCode

# 1. Tong quan van de

Cau hoi dat ra: Trong GCode cho may CNC va may in 3D, lenh "G01 X10 Y10" va "G01 x10 y10" co giong nhau khong? Chu hoa X va chu thuong x co y nghia khac nhau khong?

# 2. Tieu chuan GCode va phan biet hoa/thuong

2.1. Tieu chuan NIST / RS-274

Theo tieu chuan NIST va RS-274 — nen tang cua GCode cong nghiep — GCode KHONG phan biet chu hoa va chu thuong (case-insensitive), ngoai tru phan comment.

Vi du ba lenh sau tuong duong hoan toan:

G01 X10 Y20 F1000

g01 x10 y20 f1000

G01 x10 Y20 F1000

2.2. Kiem tra theo tung firmware / controller

| Firmware / Controller | X vs x | Ghi chu |
| --- | --- | --- |
| Marlin (may in 3D) | Nhu nhau | Case-insensitive hoan toan |
| RepRapFirmware (Duet3D) | Nhu nhau (v1.19+) | Phan biet trong quoted string |
| LinuxCNC | Nhu nhau | Theo chuan NIST RS-274 |
| Tormach | Nhu nhau | Khuyen nghi dung chu hoa |
| Aerotech Automation1 | KHAC NHAU (*) | Ten truc phan biet hoa/thuong |

# 3. Truong hop dac biet: Aerotech controller

Aerotech la hang san xuat controller chinh xac cao cap cho in chip ban dan, laser micro-machining, may do toa do CMM. Controller Aerotech Automation1 co quy tac rieng:

"The only case-sensitive part of the Aerotech G-code dialect is the name of an axis. If you have a controller with an axis named X and an axis named x, they control different axes." — Aerotech Documentation

Vi du cu the:

G1 X50       ; Di chuyen truc X (Gantry chinh) den 50mm

G1 x-0.013   ; Di chuyen truc x (Fine stage) bu -0.013mm

=> G1 X50 khac G1 x50 — hai lenh dieu khien hai truc vat ly khac nhau hoan toan!

# 4. Tai sao can hai truc cung huong? — Coarse-Fine Motion

4.1. Van de voi Gantry lon (truc X)

Gantry co hanh trinh dai (0~500mm), toc do cao, nhung luon co sai so co hoc do rung, gian no nhiet, ma sat o truot. Sai so thuc te thuong 5~20 um (micromet).
Vi du: ra lenh G1 X100 nhung gantry thuc te dung o X=100.013mm => lech 13um.

4.2. Giai phap: Fine Stage (truc x)

Fine stage la co cau chuyen dong nho, gan tren gantry, hanh trinh ngan (+-2mm) nhung do chinh xac cuc cao (~0.1um). Encoder doc sai so thuc te => controller tu dong ra lenh cho fine stage bu nguoc lai:

Gantry dung tai X=100.013, can ve X=100.000

G1 x-0.013   ; Fine stage bu -13um de dau tool dung vi tri

4.3. So sanh Coarse vs Fine

| Dac diem | Truc X (Gantry) | Truc x (Fine Stage) |
| --- | --- | --- |
| Hanh trinh | 0 ~ 500 mm | +-2 mm |
| Do chinh xac | ~10 um (tho) | ~0.1 um (tinh) |
| Toc do | Cao (m/s) | Thap (mm/s) |
| Dan dong | Linear motor/rack | Voice coil / piezo |
| Lenh GCode | G1 X100 | G1 x-0.013 |

# 5. Ung dung thuc te

- May in chip ban dan (ASML EUV): Coarse stage dich wafer silicon ~300mm, fine stage (tu truong) can chinh den 1nm. Xem: youtube.com/watch?v=1fOA85xtYxs

- Laser micro-machining: Cat, khoan chi tiet nho tren mach dien tu. Gantry di chuyen nhanh, fine stage bu sai so.

- May do toa do CMM: Dau do di chuyen tren khung lon, fine stage dam bao tiep xuc dung diem do voi sai so <1um.

- Robot pick & place linh kien: Can chinh chinh xac +-0.05mm de dat linh kien dung pad tren mach in.

Tai lieu tham khao:

- G-Code Programming

- How an ASML Lithography Machine Moves a Wafer